Brake System Plausibility Device (BSPD) – Description and Design Logic

1. Introduction – What is a BSPD

The Brake System Plausibility Device (BSPD) is a mandatory safety system required by the Formula Student / Formula SAE rulebook for vehicles with combustion engines. Its primary purpose is to detect an implausible condition where significant throttle demand and brake application occur simultaneously for a defined amount of time and to disable engine torque output in such a case.

The BSPD is an independent hardware safety circuit that operates separately from the ECU software. This ensures that even in the event of ECU malfunction, software error, or sensor signal misinterpretation, the vehicle can still be brought to a safe state.

2. Purpose and Function of the BSPD

The BSPD is designed to protect against scenarios such as:

- Throttle stuck open while the driver is braking

- Mechanical or electrical failures that cause unintended engine torque

- Driver input conflicts that persist longer than a short transient

When a defined implausible condition is detected and persists longer than the allowed time, the BSPD opens the shutdown circuit, cutting engine torque and forcing the vehicle into a safe state.

The BSPD does not attempt to control the vehicle or correct faults. Its sole function is to detect and react by disabling torque.

3. What the BSPD Checks

In this design, the BSPD evaluates three independent logical conditions:

3.1 Throttle Position Sensor (TPS) Plausibility

The throttle position signal is monitored using a comparator circuit. The signal is checked against defined voltage thresholds to ensure that it lies within the valid operating range.

If the throttle signal exceeds or falls below the acceptable limits (e.g. outside ~0.5 V to ~4.5 V), a fault signal (TPS_OUT) is generated.
This check is used only to validate signal plausibility, not to judge driver intent.

3.2 Brake Pressure Sensor (BPS) Plausibility

Similarly, the brake pressure signal is monitored using a comparator. The signal is compared against predefined thresholds to confirm that it represents a valid brake application signal.

If the brake signal is implausible, a fault output (BPS_OUT) is generated.
This check ensures that the BSPD does not rely on an invalid or floating brake signal.

3.3 Brake-Throttle Plausibility (Main BSPD Comparator)

The core BSPD function compares:

- Throttle position

- Brake pressure

This comparator determines whether high throttle demand and significant brake application occur at the same time. If this implausible condition is detected, a BSPD fault signal (BSPD_F) is asserted.

This stage represents the actual BSPD condition defined by the FSAE rules.

4. What the BSPD Does NOT Check

It is important to clearly define the limits of BSPD responsibility:

- The BSPD is not responsible for diagnosing sensor failures

- It does not replace throttle plausibility checks

- It does not validate sensor accuracy or calibration

- It does not shut down the vehicle due to a brief spike or noise

Sensor malfunctions such as:

- TPS short to 5 V

- TPS short to ground

- Broken or disconnected BPS sensor

are handled by separate plausibility circuits or the ECU, not by the BSPD logic itself.

The BSPD only reacts when valid brake and throttle signals together form an implausible driving condition.

5. Logic Architecture of the BSPD

5.1 Comparator Outputs

The following digital fault signals are generated:

- TPS_OUT – throttle signal plausibility

- BPS_OUT – brake signal plausibility

- BSPD_F – brake/throttle implausibility

Each comparator includes hysteresis and input filtering to prevent false triggering due to noise or transient disturbances.

5.2 Logic Combination (AND Logic)

All three fault signals are combined using a digital AND gate.
The AND logic ensures that:

The shutdown is triggered only when all required conditions are true simultaneously

This prevents false shutdowns caused by:

- Single sensor glitches

- Temporary noise

- Transient signal overlap

Using AND logic guarantees that the BSPD activates only for a real, sustained implausibility, which aligns with both the rule intent and good safety engineering practice.

5.3 Time Delay Requirement

The output of the AND gate feeds a monostable timer (NE555) configured to introduce a delay of approximately 500 ms.

This delay ensures that:

- Short transients do not trigger shutdown

- The condition must persist continuously

- The BSPD reacts only to sustained faults

Once the delay expires, the BSPD output opens the shutdown circuit via a relay, disabling engine torque.

6. Actuation and Shutdown Behavior

The final BSPD output controls a relay in the shutdown circuit.
When the BSPD triggers:

- The relay opens

- Engine torque is disabled

- The vehicle must be reset manually before operation can resume

This behavior is fail-safe, as loss of BSPD power or signal defaults to a shutdown state.

7. Design Philosophy and Justification

The BSPD design follows these principles:

- Independent hardware safety

- Clear separation of responsibilities

- No software dependency

- Noise-robust comparator stages

- Pure digital logic for decision making

- Time-based confirmation of faults

By separating sensor plausibility checks from the BSPD condition itself, the design avoids unnecessary shutdowns while still fully complying with the intent of the FSAE rules.

8. Conclusion

The BSPD presented in this design correctly fulfills its intended role as defined by Formula Student / Formula SAE regulations. It detects sustained brake-throttle implausibility, applies appropriate logic and timing, and safely disables engine torque when required.

The BSPD does not attempt to diagnose sensor failures or replace ECU-level plausibility checks. Instead, it provides a robust, independent, and rule-compliant safety layer that ensures safe vehicle operation under fault conditions.